实验数据

表2

| Attack scenario | Attack code / injected action | Targeted PLC object | Loss before protection | After LiTEE-PLC |
| --- | --- | --- | --- | --- |
| Physical pin hijack | Write PIOB SODR (0x400E1030) | GPIO output pin (valve) | Pressure 1200→200 kPa, Flow 800→100 L/min, Valve 50→100% | Blocked (g_rejectCount=1). MPU fault triggered. Pressure 1200 kPa, Flow 800 L/min, Valve 50% |
| False data injection | Write int_input[0..1]; Kalman filter + sliding window detection (THRESHOLD=9.647, WINDOW=5) | Pressure/Flow sensor input buffer | Pressure 1200→50 kPa, Flow 800→10 L/min. PLC reads corrupted data | Detected by Kalman filter (g_rejectCount=2). Residual > 9.647, 5 consecutive violations trigger window. Anomalous values rejected. Pressure 1200 kPa, Flow 800 L/min |
| Watchdog disable | Write WDT_MR (0x400E1454) | Watchdog timer control | WDT disabled. System loses reset-on-hang protection | Blocked (g_rejectCount=3). MPU fault triggered. WDT remains active |
| Setpoint tampering | Write int_output[0] | Pressure setpoint register | Setpoint 1000→0 kPa. PLC targets zero pressure | Rejected (g_rejectCount=4). HMAC verification failed. Setpoint remains 1000 kPa |
| I/O buffer tampering | Write bool_output[0][0..1] | Output image memory (motor/valve) | Motor 1500→3000 RPM (overspeed), Valve 50→0% (fully closed) | Blocked (g_rejectCount=5). MPU fault triggered. Motor 1500 RPM, Valve 50% |
| Control program injection | Write Flash address 0x90000 | Program code region (Flash) | Program code corrupted, invalid program hash | Blocked (g_rejectCount=6). Flash Region 2 read-only. HMAC verification valid. Original program preserved |

表3

Table 3 — 卡尔曼滤波参数调优结果

实验设置：每组参数运行 100 个正常周期（测量 FAR）+ 50 个攻击周期（测量 FNR），用 13 μs 误差计。

| 参数 | 值 | FAR | FNR | 平均扫描周期运行时 (μs) |
| --- | --- | --- | --- | --- |
| k (阈值倍数) | 1 | 54.0% | 0% | 13 |
|  | 2 | 31.0% | 0% | 13 |
|  | 3 | 9.0% | 0% | 13 |
|  | 4 | 0% | 0% | 13 |
|  | 5 | 0% | 0% | 13 |
| w (窗口大小) | 1 | 14.0% | 0% | 13 |
|  | 3 | 11.0% | 0% | 13 |
|  | 5 | 9.0% | 0% | 13 |
|  | 10 | 4.0% | 0% | 13 |
|  | 20 | 0% | 0% | 13 |

注：Runtime 由 micros() 测量，实际值 ≤ 13 μs（可能为 13 ± 1 μs），建议后续用 DWT 周期计数器精测。

最优参数推荐

- 唯一满足 FNR=0 且 FAR 最低的组合：k=4, w=5（FAR=0%, FNR=0%）

- 论文常用的 3σ 规则组合：k=3, w=5（FAR=9%, FNR=0%）

论文中可以写：经调参，我们选择 k=4, w=5（FAR=0%），或遵循 3σ 规则选择 k=3, w=5。

建议选择：k=4, w=5（FAR=0%，FNR=0%，运行时不变）。如需进一步讨论，可准备 Table 5。

Table 5: Memory overhead comparison (Arduino Due, 96 KB SRAM, 512 KB Flash)

| System | SRAM usage (bytes) | Flash usage (bytes) | Key mechanism | MPU used? |
| --- | --- | --- | --- | --- |
| LiTEE‑PLC (original) | 6,512 | 63,776 | Hardware MPU isolation | ✅ Yes |
| PISTIS_Software | 7,020 | 63,824 | Software shadow‑memory check | ❌ No |
| M2MON_SVC | 6,764 | 63,680 | SVC‑based MMIO proxy + frequency table | ❌ No |

Table 5 notes:
(1) SRAM usage includes global variables, buffers, and security-related metadata (e.g., PISTIS shadow table, M2MON frequency table).
(2) Flash usage includes code and read-only data; MPU configuration code is excluded in the two software-based variants.
(3) The observed memory gradient (PISTIS > M2MON > original) is consistent with the theoretical overhead of software-only isolation vs. hardware-assisted isolation.